BASIC PIPE CONNECTION USING VALVES, TAPS, ELBOWS, T IN HOUSE HOLD FITTINGS

AIM:

To prepare a pipe line connection with all fittings.

TOOLS REQUIRED:

1. Pipe wrench

2. Adjustable spanner

3. Measuring tape

4. Plumbing vice

FITTINGS REQUIRED:

1. Bend

2. Coupling

3. Elbow

4. T

5. Tap

6. Valve

7. End cap

8. Reducer

PROCEDURE:

1. First draw the plumbing lines sketches for the basic pipe connection.

2. Take the plumbing fittings and tools.

3. Then connect the pipe line with the plumbing fittings as per the diagram.

4. Then tighten each connection properly by fitting them in plumbing vice.

RESULT:

Thus the basic pipe connection using valve tap T, elbow, etc., is done.

PREPARATION OF MIXED PIPE MATERIAL CONNECTION FOR HOUSE HOLD APPLIANCES

AIM:

To draw and construct mixed pipe materials connection for washing machine and water        heater.

TOOLS REQUIRED:

1. Pipe wrench

2. Adjustable spanner

3. Measuring tape

4. Plumbing vice

5. Hacksaw

MATERIALS REQUIRED:

1. Bend

2. Coupling

3. Flexible pipes and hoses

4. T

5. Tap

6. Valve

7. End cap

8. Reducer

9.GI and PVC pipes

10. Union

PROCEDURE:

1. First draw the plumbing line sketches.

2. Mark the location for washing machine, water heater and pipe lines.

3. Now conncet the pipes line to the fitting as per the plumbing line sketch.

4. The water tank was filled with water and gate valve was opened slowly to supply water into pipe line.

5. Then properly tighten the connections.

RESULT:

Thus the pipe line connection to a washing machine and water heater is done.

PLANNING

AIM:

To plan the given work piece according to the size given.

MATERIALS REQUIRED:

A wooden piece of size  _________________  mm.

TOOLS REQUIRED:

1. Jack plane

2. Steel rule

3. Try square

4. Marking gauge

5. Bench vice

OPERATIONS:

1. Planning

2. Marking

3. Finishing

PROCEDURE:

- The given job is clamped in a carpentry vice and any two surfaces are planned to get right angle.

- The right angle required is to be checked by try square..

- Required size is marked on the work piece by means of using steel rule and marking gauge.

- The sizes marked on the wooden pieces are to be planned by means of wooden jack plane.

- the planned surfaces are again checked for it’s accurate size and shape and angle.

RESULT:

Thus the given job is planned and the correct dimensions size and shapes are obtained.

TEE JOINT OR MIDDLE LAP JOINT

AIM:

To make a Tee or middle lap joint on the work piece according to the given dimensions.

MATERIALS REQUIRED:

A wooden piece of size  _________________  mm.

TOOLS REQUIRED:

1. Jack plane

2. Steel rule

3. Try square

4. Marking gauge

5. Bench vice

6. Rip saw

7. Firmer chisel

8. Mallet

OPERATIONS:

1. Planning

2. Marking

3. Cutting

4. Chiseling

5. Finishing

PROCEDURE:

1. The given job is clamped in carpentry vice and any two surfaces are planned to get right angle.

2. The right angle required is to be checked by try square.

3. Now all the four surfaces of the wooden pieces are planned to get the smoother and finished surface.

4. Then the job is cut into two halves.

5. The different markings required for making Tee Halving joints are to be marked on the work piece by means of using steel rule and marking gauges.

7. One half of the work piece is taken using saw and chisel to remove the unwanted portion as per the dimension.

8. The other half of the work piece is also produced by means of repeating the above procedure.

9. The two pieces are to be assembled to get a proper joint.

10. The job is again checked for it’s accurate shape and size  using try square and steel rule.

RESULT:

Thus the Tee or middle lap joint is made on the work piece according to the given dimensions.

MORTISE AND  TENON JOINT

AIM:

To make a mortise and tenon joint on the work piece according to the given dimensions.

MATERIALS REQUIRED:

A wooden piece of size  _________________  mm.

TOOLS REQUIRED:

1. Jack plane

2. Steel rule

3. Try square

4. Marking gauge

5. Bench vice

6. Rip saw

7. Firmer chisel

8. Mallet

OPERATIONS:

1. Planning

2. Marking

3. Cutting

4. Chiseling

5. Finishing

PROCEDURE:

1. The given job is clamped in carpentry vice and any two surfaces are planned to get right angle.

2. The right angle required is to be checked by try square.

3. Now all the four surfaces of the wooden pieces are planned to get the smoother and finished surface.

4. Then the job is cut into two halves.

5. The different markings required for making Tee Halving joints are to be marked on the work piece by means of using steel rule and marking gauges.

7. One half of the work piece is taken using saw and chisel to remove the unwanted portion as per the dimension.

8. The other half of the work piece is also produced by means of repeating the above procedure.

9. The two pieces are to be assembled to get a proper joint.

10. The job is again checked for its accurate shape and size using try square and steel rule.

RESULT:

Thus the mortise and tenon joint is made on the work piece according to the given dimensions.

HALF LAP JOINT

AIM:

To make a half lap joint on the work piece according to the given dimensions.

MATERIALS REQUIRED:

A wooden piece of size  _________________  mm.

TOOLS REQUIRED:

1. Jack plane

2. Steel rule

3. Try square

4. Marking gauge

5. Bench vice

6. Rip saw

7. Firmer chisel

8. Mallet

OPERATIONS:

1. Planning

2. Marking

3. Cutting

4. Chiseling

5. Finishing

PROCEDURE:

1. The given job is clamped in carpentry vice and any two surfaces are planned to get right angle.

2. The right angle required is to be checked by try square.

3. Now all the four surfaces of the wooden pieces are planned to get the smoother and finished surface.

4. Then the job is cut into two halves.

5. The different markings required for making Tee Halving joints are to be marked on the work piece by means of using steel rule and marking gauges.

7. One half of the work piece is taken using saw and chisel to remove the unwanted portion as per the dimension.

8. The other half of the work piece is also produced by means of repeating the above procedure.

9. The two pieces are to be assembled to get a proper joint.

10. The job is again checked for it’s accurate shape and size  using try square and steel rule.

RESULT:

Thus the half lap joint is made on the work piece according to the given dimensions.

BUTT JOINT

AIM:

To make a butt joint on the given mild steel plate by arc welding.

MATERIALS REQUIRED:

Two mild steel plate’s size ________________mm

TOOLS REQUIRED:

- Try square

- File

- Steel rule

- Electrode

- Tongs

- wire brush

- Chipping hammer

- Gloves

PROCEDURE:

1. Two mild steel plates of size___________________  is taken.

2. The metal plates are cleaned by wire brush and are wiped thoroughly.

3. One edge of each metal plate are filed smoothly.

4. The metal plates are placed to required position.

5. Suitable electrodes and the current required for welding are selected. The electrode is held in the holder.

6. The hand shield must be used during arc welding.

7. The electrode is brought near the work piece with an air gap of about 3mm.

8. The ends are welded first then slowly welding is done between the two ends.

9. After welding process is over by using chipping hammer, the slag over the welded portion is removed then it is cleaned with the help of wire brush.

10. Now the joint is finished.

RESULT:

Thus the two mild two plates are welded to get a butt joint

LAP JOINT

AIM:

To make a lap joint on the given mild steel plate by arc welding.

MATERIALS REQUIRED:

Two mild steel plate’s size  ________________mm

TOOLS REQUIRED:

- Try square

- File

- Steel rule

- Electrode

- Tongs

- wire brush

- Chipping hammer

- Gloves

PROCEDURE:

1. Two mild steel plates of size___________________ is taken.

2. The metal plates is cleaned by wire brush and are wiped thoroughly.

3. One edge of each metal plate is filed smoothly.

4. The metal plates are placed to required position.

5. Suitable electrodes and the current required for welding are selected. The electrode is held in the holder.

6. The hand shield must be used during arc welding.

7. The electrode is brought near the work piece with an air gap of about 3mm.

8. The ends are welded first then slowly welding is done between the two ends.

9. After welding process is over by using chipping hammer, the slag over the welded portion is removed then it is cleaned with the help of wire brush.

10. Now the joint is finished.

RESULT:

Thus the two mild two plates are welded to get a lap joint

TEE JOINT

AIM:

To make a Tee joint on the given mild steel plate by arc welding.

MATERIALS REQUIRED:

Two mild steel plates ________________mm

TOOLS REQUIRED:

- Try square

- File

- Steel rule

- Electrode

- Tongs

- wire brush

- Chipping hammer

- Gloves

PROCEDURE:

1. Two mild steel plates of size___________________  is taken.

2. The metal plates are cleaned by wire brush and are wiped thoroughly.

3. One edge of each metal plate is filed smoothly.

4. The metal plates are placed in Tee required position.

5. Suitable electrodes and the current required for welding are selected. The electrode is held in the holder.

6. The hand shield must be used during arc welding.

7. The electrode is brought near the work piece with an air gap of about 3mm.

8. The ends are welded first then slowly welding is done between the two ends.

9. After welding process is over by using chipping hammer, the slag over the welded portion is removed then it is cleaned with the help of wire brush.

10. Now the joint is finished.

RESULT:

Thus the two mild two plates are welded to get a Tee joint

TRAY

AIM

To make a tray out of the given sheet with specified dimensions.

TOOLS REQUIRED

1.Bench vice

2.Try square

3.Steel rule

4.Divider

5.Snap

6.Mallet

7.Hand shearing

8.Dot punch

9.Scriber

MATERIALS REQUIRED

G.I. Sheet of suitable size

PROCEDURE:

1.Draw the lay out of the bottom and body of the tray

2.The edges are marked clearly on the sheet metal using pencil

3.The given sheet is cut to the required shape and size using hand shears

4.The marked lines are folded with the help of mallet and required stakes

5.After folding on the all sides it resembles a tray

6.The folds are made with dot punch on the marked portions for riveting

RESULT:

Thus the required tray is finished from the given sheet

FUNNEL

AIM

To make a Funnel out of the given sheet with specified dimensions.

TOOLS REQUIRED

1.Bench vice

2.Try square

3.Steel rule

4.Divider

5.Snap

6.Mallet

7.Hand shearing

8.Dot punch

9.Scriber

MATERIALS REQUIRED

G.I. Sheet of suitable size

PROCEDURE;

1.Draw the lay out of the funnel on the given sheet metal (Part I and Part II)

2.The angle subtended by the cone when opened for lay out is calculated by using

The relation

Angle =3.14xDiameter of the base

Slant height

3The edges are marked clearly on the sheet metal using pencil

4.The given sheet is cut to the required shape and size using hand shears

4.The marked lines are folded with the help of mallet and required stakes

5.After folding on the all sides it resembles a funnel

6.The folds are made with dot punch on the marked portions for riveting

RESULT

Thus the funnel is finished from the given sheet

FACING AND PLAIN TURNING

AIM

To get a required shape and size of a given work piece by Facing and plain Turning operations done in lathe.

TOOLS REQUIRED:

1.Single point cutting tool

2.Vernier Caliper

3.Tool post key

4.Steel rule

5.Scriber

MATERIALS REQUIRED

Mild steel rod of diameter ___________and length______

PROCEDURE

1 First the given work piece is checked for its dimensions

2 The work piece is fitted in the chuck properly and tighten by the chuck key

3 Ensure the center of work piece.

4 Hold the cutting tool in the Tool post ,check the tool tip whether it is  coincide

With tail stock spindle tip for centering and then tighten the nuts using tool post

Key.

5 Facing operation is done with cutting tool by many it perpendicular to lathe axis.

6 It is done on both sides of workpiece up to the required length.

7 Then turning operation is done with cutting tool which removes the upper part of

The workpiece to reduce the diameter up to the dimensions required.

8 Then   remove the work piece from the chuck

9 Atlast the dimensions of workpiece are checked.

RESULT

Thus the required shape and size of a given work piece is done on lathe by Facing and

Plain Turning  Operation.

STEP TURNING

AIM

To get a required shape and size of a given work piece by Step Turning operations done in lathe.

TOOLS REQUIRED:

1.Single point cutting tool

2.Vernier Caliper

3.Tool post key

4.Steel rule

5.Scriber

MATERIALS REQUIRED

Mild steel rod of diameter ___________and length______

PROCEDURE

1 First the given work piece is checked for its dimensions

2 The work piece is fitted in the chuck properly and tighten by the chuck key

3 Ensure the center of work piece.

4 Hold the cutting tool in the Tool post ,check the tool tip whether it is  coincide

With tail stock spindle tip for centering and then tighten the nuts using tool post

Key.

5 Facing operation is done with cutting tool by many it perpendicular to lathe axis.

6 It is done on both sides of workpiece up to the required length.

7 Then turning operation is done with cutting tool which removes the upper part of

The workpiece to reduce the diameter up to the dimensions required.

8 Then   remove the work piece from the chuck

9 Atlast the dimensions of workpiece are checked.

RESULT

Thus the required shape and size of a given work piece is done on lathe by Step

Turning  Operation.

STEP TURNING

AIM

To get a required shape and size of a given work piece byTaper Turning operations

done in lathe.

TOOLS REQUIRED:

1.Single point cutting tool

2.Vernier Caliper

3.Tool post key

4.Steel rule

5.Scriber

MATERIALS REQUIRED

Mild steel rod of diameter ___________and length______

FORMULA:

Taper angle=tan-1 D-d

2L

D=Diameter of big end

d=Diameter of small end

L=Length between two ends

PROCEDURE

- First the given work piece is checked for its dimensions

- The work piece is fitted in the chuck properly and tighten by the chuck key

- Ensure the center of work piece.

- Hold the cutting tool in the Tool post , check the tool tip whether it is  coincide

with tail stock spindle tip for centering and then tighten the nuts using tool post

key.

- Facing operation is done with cutting tool by many it perpendicular to lathe axis.

- It is done on both sides of work piece up to the required length.

- Then turning operation is done with cutting tool which removes the upper part of

- The work piece to reduce the diameter up to the dimensions required.

- Next the Taper turning is done on the work piece according to the taper angle

- Calculated

- The compound rest base is swiveled and set at calculated taper angle

- Cutting tool is move towards at an angle to the lathe axis

- Tool is removed by compound rest hand wheel up to required angle.

- Then remove the work piece from the chuck

- At last the dimensions of work piece are checked.

RESULT

Thus the required shape and size of a given work piece is done on lathe by Taper

Turning Operation.

STEPPED CONE PULLEY

AIM:

To prepare the green sand mould for the given stepped cone pulley

TOOLS REQUIRED:

Moulding boxes, Moulding board, Shovel, Peen and round rammer, Strike off bar,

Trowel, Slick, Sprue pins, Vent wire, Brush, Lifter, Swab, Cleaner and gate cutter.

PROCEDURE:

1. Select a suitable moulding box.

2 Here a single piece pattern is used so place one half of the pattern on a moulding

Board.

3. Place the drag box around the pattern and keep 20mm layer of facing sand around the pattern.

4. Fill the track with loose sanding sand.

5. Ram the sand by rammer around the pattern uniformly.

6. Fill the sand up to the top of box. Strike off the excess sand by using strike off bar and bring it at the same level of the flask height. This completes the drag.

7. Make vent hole in the sand.

8. Now the drag is tilted upside down.

9. Sprinkle parting sand over the top of the drag.

10. Place copes over the rammed drag.

11. Place sprue pin and riser pin in position then fill the cope with the moulding sand and ram it uniformly using a rammer.

12. Strike off the excess sand from the cope.

13. Make vent holes in the cope using vent wire.

14. Remove sprue pin and raiser pin and cut pouring basin at the top to pour the molten metal.

15. Separate the cope and drag.

16. Finish and smooth the top surfaces of cope and drag with trowel.

17. With draw the pattern from the mould by using drawspike.

18. Cut the gate connecting the sprue basin with mould cavity.

19. Carry out the repair work in the mould if necessary.

20. Apply mould cavity coating with swab.

21. Now assemble the two mould boxes in correct positions and place the pouring the weight.

22. Now the mould is ready for pouring.

RESULT:

Thus the mould for step cone pulley is made.

GEAR WHEEL

AIM:

To prepare the green sand mould for the given gear wheel.

TOOLS REQUIRED:

Moulding boxes, Moulding board, Shovel, Peen and round rammer, Strike off bar,

Trowel, Slick, Sprue pins, Vent wire, Brush, Lifter, Swab, Cleaner and gate cutter.

PROCEDURE:

1. Select a suitable moulding box.

2 Here a single piece pattern is used so place one half of the pattern on a moulding

board.

3. Place the drag box around the pattern and keep 20mm layer of facing sand around the pattern.

4. Fill the track with loose sanding sand.

5. Ram the sand by rammer around the pattern uniformly.

6. Fill the sand up to the top of box. Strike off the excess sand by using strike off bar and bring it at the same level of the flask height. This completes the drag.

7. Make vent hole in the sand.

8. Now the drag is tilted upside down.

9. Sprinkle parting sand over the top of the drag.

10. Place copes over the rammed drag.

11. Place sprue pin and riser pin in position then fill the cope with the moulding sand and ram it uniformly using a rammer.

12. Strike off the excess sand from the cope.

13. Make vent holes in the cope using vent wire.

14. Remove sprue pin and raiser pin and cut pouring basin at the top to pour the molten metal.

15. Separate the cope and drag.

16. Finish and smooth the top surfaces of cope and drag with trowel.

17. With draw the pattern from the mould by using drawspike.

18. Cut the gate connecting the sprue basin with mould cavity.

19. Carry out the repair work in the mould if necessary.

20. Apply mould cavity coating with swab.

21. Now assemble the two mould boxes in correct positions and place the pouring the weight.

22. Now the mould is ready for pouring.

RESULT:

Thus the mould for gear wheel is made.

SQUARE FITTING

AIM:

To make a square fitting on the given work piece with proper dimension.

MATERIALS REQUIRED:

Mild steel, plates of _____________

TOOLS REQUIRED:

1. Bench vice

2. Try square

3. Steel rule

4. Surface plate

5. Surface gauge

6. Hacksaw

7. Angle plate

8. Dot punch

9. Scriber

OPERATIONS:

- Right angle the given work piece.

- Darken the face of work by chalk.

- Marking

- Punching

- Cutting

- Filing to finish.

PROCEDURE:

- After checking the dimensions using steel rule the job is fixed on a vice and the sides are filed using a flat file so that they are at right angles.

- Chalk is applied on the work piece.

- With the help of caliper surface plate and steel rule the given dimensions are marked on the work piece and punching is done using dot punch.

- By using the hacksaw the unwanted portions are removed and the corresponding surfaces are filed and smoothened to get require dimension and the accuracy is checked by a try square.

- Finally when these two pieces are fitted they form a square fitting.

RESULT:

Thus the square fitting is obtained on the given work piece.

‘V’ FITTING

AIM:

To make a ‘V’ fitting on the given work piece with proper dimension.

MATERIALS REQUIRED:

Mild steel, plates of _____________

TOOLS REQUIRED:

1. Bench vice

2. Try square

3. Steel rule

4. Surface plate

5. Surface gauge

6. Hacksaw

7. Angle plate

8. Dot punch

9. Scriber

OPERATIONS:

- Right angle the given work piece.

- Darken the face of work by chalk.

- Marking

- Punching

- Cutting

- Filing to finish.

PROCEDURE:

- After checking the dimensions using steel rule the job is fixed on a vice and the sides are filed using a flat file so that they are at right angles.

- Chalk is applied on the work piece.

- With the help of caliper surface plate and steel rule the given dimensions are marked on the work piece and punching is done using dot punch.

- By using the hacksaw the unwanted portions are removed and the corresponding surfaces are filed and smoothened to get require dimension and the accuracy is checked by a try square.

- Finally when these two pieces are fitted they form a square fitting.

RESULT:

Thus the ‘V’ fitting is obtained on the given work piece.

SQUARE FILING

AIM:

To make a square filing on the given work piece with proper dimension.

MATERIALS REQUIRED:

Mild steel, plates of _____________

TOOLS REQUIRED:

1. Bench vice

2. Try square

3. Steel rule

4. Surface plate

5. Surface gauge

6. Hacksaw

7. Angle plate

8. Dot punch

9. Scriber

OPERATIONS:

- Right angle the given work piece.

- Darken the face of work by chalk.

- Marking

- Punching

- Cutting

- Filing to finish.

PROCEDURE:

- After checking the dimensions using steel rule the job is fixed on a vice and the sides are filed using a flat file so that they are at right angles.

- Chalk is applied on the work piece.

- With the help of caliper surface plate and steel rule the given dimensions are marked on the work piece and punching is done using dot punch.

- By using the hacksaw the unwanted portions are removed and the corresponding surfaces are filed and smoothened to get require dimension and the accuracy is checked by a try square.

- Finally when these two pieces are fitted they form a square fitting.

RESULT:

Thus the square filing is obtained on the given work piece.